# Meltdown & Spectre attacks

- When we read from a memory address multiple times, the second access is usually faster than the first access, what is the reason?

- When the memory address was read multiples times, the memory address will be loaded into CPU cache, therefore the second access is faster.

- Regarding the Flush+Reload technique used in the book, why do we use an array of size 256*4096, why not use an array of size 256?

- We use array [i*4096] so no two elements will be saved in the same cache blocks, because 4096 is significantly larger than typical cache block 64 bits.

- How do we use CPU cache as a side channel to send out a number 89?

- Flush the array

- Int array[89*4096+1024]

- Reload the array

- We have defined an array of size 1024 (bytes). If the CPU cache size is 128 bytes (i.e., when memory address x is accessed, the memory from x to x+127 will be cached by the CPU). Please describe how many distinct values we can send out using this array and the Flush+Reload technique.

- 1024/128=8

- A secret number is stored at the kernel address 0xfb102000. The following user-level
program tries to access and print out this number. What is going to happen?

#include <stdio.h>
int main(){

char *kernel_data_addr = (char*)0xfb102000;	➀

char kernel_data = *kernel_data_addr;		②

printf("I have reached here.\n"); 		③

return 0;

}

The program will crash, based on Segmentation Fault, this because it’s a user-level program which cannot access the kernel

- A secret number is stored at the kernel address 0xfb102000. You are trying to print it
out. Please use plain English to describe how you can do this.

- We need to catch the Segmentation fault in order to prevent the program to crash

- We need to use out-of-order execution, we need a line that cause the exception

Kernel_data=*(char*)kernel_data_addr;

So that the next array will not be executed

array[7*4096+1024] += 1;

But because of out_of_order execution, it will run before the access check is ran so it is later deleted but the cache isn’t. then you use the CPU access time to find the secret value at the location in the memory and print it out.

- To improve the success rate of the Meltdown attack, it is better that the targeted kernel memory has already been cached by CPU. Why?

- Loading the data kernel into register will be much faster, and we may be able to get critical instruction

- If CPU does not do out-of-order execution, can we still launch the Meltdown attack? What is the downside?

- Out-of-order execution

+ Technique: allow a processor to execute the instructions in a different order than the program specifies.

+ improve performance by: exploit parallelism, stalls, hiding latency

- Downside: worsen the system performance

- KAISER (Kernel Address Isolation to have Side-channels Efficiently Removed) can be used to defeat the Meltdown attack. Its main idea is to not map kernel memory in the user space. Explain why this idea works.

- In the Spectre attack, why do we need to train the CPU?

- So that the instructions in the speculative branch are more likely to be picked out for out-of-order execution.

- If CPU does not do out-of-order execution, can we still launch the Spectre attack?

- No, since the branch execution is carry out only when the condition check is finish.

- In the Spectre attack scenario, the protected memory and the attacker program are in the same process, why can’t attackers directly access the protected memory?

- The protected memory is only accessible through a sandbox function that check out the address range that we are allow to access.

- When we run the SpectreAttack.c program listed in the book, why do we always see array[0*4096 + DELTA] in the cache?

- Block[0] is adjacent to other variables , thus may be cached in other activities.

- Your program runs in a sandbox, and it is allowed to access the first 100 elements of a
protected buffer; the access has to go through the following sandbox API. The address
of the buffer is at 0xbfff0200. There is a secret number stored at 0xbfff0100, but
your program cannot access it due to the sandbox protection. Can you get this secret
number? If so, please describe how.

int low = 0;

int high = 100;
int restrictedAccess(int x){

if (x > low && x < high)

return buffer[x];
else return 0;

}